![InfiIoT](https://infiiot.com/static/images/logo.png)

- Home

- Platform
Features Request for Quote!

- Resources
Getting Started Documentation - Hardware Documentation - Software Community Forum Blog Help Center FAQ

- About Us
Know About Us! Contact Us! Privacy Policy Terms of Service

- Login

cURL ESP Python

- Introduction  - Methods to send/upload data to\\
 Infi-IoT Platform

  - How\\
 Infi-IoT Works?

  - Device\\
 authentication

  - Protocols

- HTTP  - HTTP Methods

  - HTTP Request

- Infi-IoT API\\
URLs  - Our URLs:

  - Why use HTTPs?

- Send data  - Send\\
 data to a device

  - Send\\
 data to a variable

- Retrieve data  - Request\\
 structure:

  - Expected\\
 Response:

- Response Codes

- Limits

- MQTT  - MQTT\\
 Architecture

  - MQTT messages

  - Quality\\
 of Service(QOS)

- Copyright © Infi-IoT 2020.

- All Rights Reserved.

# Introduction

Welcome to the Infi-IoT hardware documentation. This documentation will help you learn how to integrate
your hardware devices using our supported communications protocols ie. HTTPS, and MQTT.

We use REST API for better performance and scalability. The purpose of this section is to help you
understand what happens in the backstage when communicating with Infi-IoT, so you can replicate this in
your firmware.

Please note that in all the example codes below will not use our libraries, so that you understand what
happens in the backend and it is easier for you to replicate on your side.

## Methods to send/upload data to Infi-IoT  Platform

Prerequisites:

- If you don't have an Infi-Iot subscription, create one for free before you begin.

- A device that meets the following requirements:

  - Your device must be able to communicate by using HTTP or MQTT protocols.

  - The device messages must conform to the Infi-IoT Platform message payload requirements.

For more Information on Infi-IoT Platform payload requirements see csv requirements!

.csv requirements here.

Infi-IoT enables you to ingest high volumes of telemetry from your IoT devices into the cloud for storage
or analysis. Following are the ways you can add data to the cloud:

- Using Our Libraries or APIs:

1. If you have a device able to communicate by using HTTP or MQTT protocols, you can use our
   libraries which are designed for different hardwares and tools.

2. You can use our strong API service and perform all tasks by including our API into your apps
   or programs

- Using .csv file: If you have been collecting and storing data for a certain time and wish to
analyze your data using our services then you can import logged data onto the Infi-IoT Platform.
Using .csv file in the format specified
by us to import data to your account using the import data section. You can Import data for a
single variable as well as for the whole device.

## How Infi-IoT Works?

Every time a device updates a sensor value in a variable, firstly user is authenticated using Auth-Token
Key, and if the user is recognized a data-point is created. Infi-IoT stores data that come from your
devices inside variables and
these data points have corresponding timestamps.

Each dot contains Value and timestamp.

### Values

A numerical value. Infi-IoT accepts up to 16 floating-point length numbers.

{"value" : 34.87654974}

Super Tip: It is mandatory to insert sensor data into your data payload.

### Timestamps

A timestamp is a digital record of the time of occurrence of a particular event. We use ISO 8859 format
for timestamps

{"timestamp" : 2020-05-07T09:25:24+0000}

The above timestamp corresponds to Thursday, September 20, 2018, at 2:30:24 PM.

Super Tip: If not specified, then our servers will assign timestamps upon data
payload reception.

### Data Illustration:

## Device authentication

Infi-IoT uses Auth (Authentication) Token key to authorize your device to ingest data inside your
Infi-IoT account.

Authentication token keys are UUID 4 format keys that are generated when you register your Infi-IoT
account. You can find your Auth token Key

Super Tip: Keep the authentication Token keys physically safe. This will prevent a
malicious device masquerade as a registered device.

## Protocols

Infi-IoT supports two protocols for device connection and communication: MQTT and HTTP. Devices
communicate with Infi-IoT Platform across a "bridge" — either the MQTT bridge or the HTTP
bridge.

- MQTT is a standard publish/subscribe protocol that's mostly used and supported by embedded
devices, and is additionally common in machine-to-machine interactions.

- HTTP is a "connectionless" protocol: with the HTTP bridge, devices do not maintain a
connection to the Infi-IoT Platform. Instead, they send requests and receive responses. Infi-IoT
Core supports HTTP 1.1 only (not 2.0).

The following table compares how the two protocols work on Infi-IoT Platform:

| MQTT | HTTP |
| --- | --- |
| MQTT bridge | HTTP bridge |
| Device connection is maintained | Connectionless (request/response) |
| Full-duplex TCP connection | Half-duplex TCP connection |
| A token is sent within the password field of the CONNECT message | A token is sent in the Authorization header of the HTTP request |
| Telemetry events are pushed to Cloud Pub/Sub | Telemetry events are pushed to Pub/Sub |
| The device connection status is reported | No device connection status reported |
| Messages are propagated via subscriptions | Device configurations must be explicitly requested (via polling) |
| Latest messages is always received by devices on a subscription | Devices can specify that only newer configurations should be received |
| messages are acknowledged (ACKed) when using QoS 1 | No explicit ACK for device configurations |
| Last device heartbeat time (active time) is retained | No device heartbeat data |

Few more general features of each protocol are as follows:

- MQTT

  - Lower bandwidth usage

  - Lower latency

  - Higher throughput

  - Supports raw binary data

- HTTP

  - Easy to get started; simple curl commands

  - Fewer firewall issues

  - Binary data should be base64-encoded, which requires more network and CPU resources

Super Tip: If you're not sure which protocol is best for your use cases, start with
HTTP to get familiar with Cloud IoT Core, and then switch to MQTT if needed.

# HTTP

As seen earlier, HTTP is Connectionless (request/response) and Half-duplex TCP connection-based
communication protocol

This section explains how devices can use the HTTP bridge to communicate with the Infi-IoT Platform. For
general information about HTTP and MQTT, see Protocols

Be sure to refer to the API documentation-\\
Software for full details about each method described in this section.

HTTP is a request/response protocol, this means that every request that you make is answered by the
server. This response includes a number (response code) and a body. For example, when you make a request
to retrieve information from a
webpage "(e.g. google.com)", you build a GET request ( GET http://www.google.com
). If the request is correct, the server will typically return a 200 response code, along with the
information to render a webpage
for you.

Likewise, when your device sends a request to Infi-Iot requesting last value for a variable, the request
is processed and Infi-IoT sends back a response that contains a status code and a body section
containing the data (value and timestamp)
related to your device variable.

## HTTP Methods

You can think of this as the verb that tells the server what action to perform on a resource. The two
most common HTTP request methods you'll see are GET and POST. When you think about retrieving
information, think GET, and when you
think about putting the information to a server, think POST.

We prefer POST methods for all tasks for security reasons because even when you want to retrieve the last
value of your sensor data you will need to send your Token Key via URL(if GET Method is used) which is
quite insecure. So keep in
mind even when you want to retrieve data you use the POST method so that your Token is transferred
securely to our servers.

## HTTP Request

An HTTP request also needs the parameters below:

- Host: Specifies the server you will be making HTTP requests to. For example,
www.infiiot.com

- Path: This is typically the remaining portion of the URL that specifies the
resource you want to consume, be it a variable or a device. For example, if an API endpoint is:
https://infiiot.com/apis/v1.0/get-Devices
then the path would be /apis/v1.0/get-Devices

For more information about the URL refer here.

- Headers: HTTP headers allow the client and the server to send additional
information during the request/response HTTP cycle. Headers are colon-separated name-value pairs
that are sent in plain text.

Headers define the operating parameters of the HTTP request such as authentication, User-Agent,
Content-Type, Content-Length, etc.

- Body/payload: In the case of POST and PATCH requests, this is the data sent by your
device to the server. GET requests typically do not have a body because they are meant to request
data, not to send data. But as we
prefer POST we will always have Body section.

Bodies can be broadly divided into two categories: Single-resource bodies, and Multiple-resource bodies.
We require Single-resource bodies consisting of one single file, defined by the two headers:
Content-Type and Content-Length.

For more information about body refer here.

We use JSON format (JavaScript Object Notation) Content-type. JSON is a typical HTTP data type, it is a
collection of name/value pairs. In various programming languages, this is treated as an object, record,
struct, dictionary, hash table,
keyed list, or associative array. It is also human-readable and language independent. An example of a
JSON data type that InfiIoT accepts can be referenced below:

{"variable": "temperature", "data": {"value":10, "timestamp":1534881387000}}

A typical HTTP request to Infi-IoT should be set as below:

POST {PATH} HTTP/1.1

Host: {HOST}

User-Agent: {USER_AGENT}

X-Auth-Token: {TOKEN}

Content-Type: application/json

Content-Length: {PAYLOAD_LENGTH}

{PAYLOAD}

Where:

- {PATH}: Path to the resource to consume. For example,
/apis/v1.0/get-Variable to get a variable information.

- {HOST}: Host URL. Example: infiiot.com

- {USER\_AGENT}: An optional string used to identify the type of client, be it by
application type, operating system, software vendor, or software version of the requesting user
agent. Examples: ESP8266/1.0, Particle/1.2

- {TOKEN}: Unique key that authorizes your device to ingest data inside your Infi-IoT
account.

- {PAYLOAD\_LENGTH}: The number of characters in your payload. Example: The payload
{"variable": "temperature", "value": 20} will have a
content-length of 38.

- {PAYLOAD}: Data to send. Example: {"variable": "temperature",                 "value": 20}

# Infi-IoT API URLs

API access can be over HTTP or secure HTTP (HTTPs).

Note: We strongly advise to use HTTPs to make sure your data travels encrypted,
avoiding the exposure of your API token and/or sensor data.

## Our URLs:

| Using | Url | Port |
| --- | --- | --- |
| HTTP | https://infiiot.com | 80 |
| HTTPs | https://infiiot.com | 443 |
| IP | 127.0.0.1 | 80/443 |

## Why use HTTPs?

As requests can contain the Token keys, which uniquely identifies you to the server, so if someone else
copied this Token keys, they could craft a request to the server and pose as your client, and thereby
automatically be logged in without
even having access to your username or password.

This is where Secure HTTP, or HTTPS, helps. A resource that's accessed by HTTPS will start with
https:// instead of http://, and usually be displayed with a lock icon in most
browsers: With HTTPS every request/response
is encrypted before being transported on the network. This means if a malicious hacker sniffed out the
HTTP traffic, the information would be encrypted and useless.

# Send data

Using our APIs you can send data to the platform in two ways:

- Send data to a device

- Send data to a variable

If you are looking for more advanced API capabilities, such as editing devices, deleting devices and,
many more, please refer to Infi-IoT\\
Documentation-hardware.

## Send data to a device

POST https://infiiot.com/api/v1.0/update-Device

Request structure:

POST /api/v1.6/update-Device HTTP/1.1

Host: {Host}

User-Agent: {USER_AGENT}

X-Auth-Token: {TOKEN}

Content-Type: application/json

Content-Length: {PAYLOAD_LENGTH}

{PAYLOAD}

Expected Response:

HTTP/1.1 200 OK

Server: nginx

Date: Tue, 04 Sep 2018 22:35:06 GMT

Content-Type: application/json

Transfer-Encoding: chunked

Vary: Cookie

Allow: GET, POST, HEAD, OPTIONS

{PAYLOAD_LENGTH_IN_HEXADECIMAL}

{"{VARIABLE_LABEL}":
[{"status_code":
201}]}

0

You can keep the device-name as desired. If the device does not exist in your Infi-IoT Account, it will
be automatically created. Device labels can contain any alphanumerical characters with or without blank
spaces.

To set the values of the variable, just create a JSON data payload as below

Send a Single data-point

curl -X POST -H "X-Auth-Token: BBFF-Rfcgaxns6HlVb155WA0RhSY85xNDmB" -H "Content-Type: application/json" -d '{"temperature": 27}' https://infiiot.com/api/v1.0/devices/update-device

/*************************************************************************************************    
    * This Example sends harcoded data to Ubidots and serves as example for users that have devices    
    * based on ESP8266 chips    
    *    
    * This example is given AS IT IS without any warranty    
    *    
    * Made by Jose García., adapted from the original WiFiClient ESP8266 example    
    *************************************************************************************************/    
         
    /********************************    
    * Libraries included    
    *******************************/    
         
    #include <ESP8266WiFi.h>    
         
    /********************************    
    * Constants and objects    
    *******************************/    
    char const * SSID_NAME = "..."; // Put here your SSID name    
    char const * SSID_PASS = "...."; // Put here your password    
         
    char* TOKEN = ""; // Put here your TOKEN    
         
    char* DEVICE_LABEL = "truck"; // Your Device label    
         
    /* Put here your variable's labels*/    
    char const * VARIABLE_LABEL_1 = "speed";    
         
    /* HTTP Settings */    
    char const * HTTPSERVER = "infiiot.com";    
    const int HTTPPORT = 80;    
    char const * USER_AGENT = "ESP8266";    
    char const * VERSION = "1.0";    
         
    WiFiClient clientinfi;    
         
    /********************************    
    * Auxiliar Functions    
    *******************************/    
         
    void SendToInfiiot(char* payload) {    
         
    int contentLength = strlen(payload);    
         
    /* Connecting the client */    
    clientinfi.connect(HTTPSERVER, HTTPPORT);    
         
    if (clientinfi.connected()) {    
      /* Builds the request POST - Please reference this link to know all the request's structures https://ubidots.com/docs/api/ */    
      clientinfi.print(F("POST /api/v1.0/update-device"));    
      clientinfi.print(DEVICE_LABEL);    
      clientinfi.print(F(" HTTP/1.1\r\n"));    
      clientinfi.print(F("Host: "));    
      clientinfi.print(HTTPSERVER);    
      clientinfi.print(F("\r\n"));    
      clientinfi.print(F("User-Agent: "));    
      clientinfi.print(USER_AGENT);    
      clientinfi.print(F("/"));    
      clientinfi.print(VERSION);    
      clientinfi.print(F("\r\n"));    
      clientinfi.print(F("X-Auth-Token: "));    
      clientinfi.print(TOKEN);    
      clientinfi.print(F("\r\n"));    
      clientinfi.print(F("Connection: close\r\n"));    
      clientinfi.print(F("Content-Type: application/json\r\n"));    
      clientinfi.print(F("Content-Length: "));    
      clientinfi.print(contentLength);    
      clientinfi.print(F("\r\n\r\n"));    
      clientinfi.print(payload);    
      clientinfi.print(F("\r\n"));    
         
      Serial.println(F("Making request to Ubidots:\n"));    
      Serial.print("POST /api/v1.0/update-device");    
      Serial.print(DEVICE_LABEL);    
      Serial.print(" HTTP/1.1\r\n");    
      Serial.print("Host: ");    
      Serial.print(HTTPSERVER);    
      Serial.print("\r\n");    
      Serial.print("User-Agent: ");    
      Serial.print(USER_AGENT);    
      Serial.print("/");    
      Serial.print(VERSION);    
      Serial.print("\r\n");    
      Serial.print("X-Auth-Token: ");    
      Serial.print(TOKEN);    
      Serial.print("\r\n");    
      Serial.print("Connection: close\r\n");    
      Serial.print("Content-Type: application/json\r\n");    
      Serial.print("Content-Length: ");    
      Serial.print(contentLength);    
      Serial.print("\r\n\r\n");    
      Serial.print(payload);    
      Serial.print("\r\n");    
    } else {    
      Serial.println("Connection Failed ubidots - Try Again");    
    }    
         
    /* Reach timeout when the server is unavailable */    
    int timeout = 0;    
    while (!clientinfi.available() && timeout < 5000) {    
      timeout++;    
      delay(1);    
      if (timeout >= 5000) {    
        Serial.println(F("Error, max timeout reached"));    
        break;    
      }    
    }    
         
    /* Reads the response from the server */    
    Serial.println(F("\nUbidots' Server response:\n"));    
    while (clientinfi.available()) {    
      char c = clientinfi.read();    
      //Serial.print(c); // Uncomment this line to visualize the response on the Serial Monitor    
    }    
         
    /* Disconnecting the client */    
    clientinfi.stop();    
    }    
         
    /********************************    
    * Main Functions    
    *******************************/    
         
    void setup() {    
    Serial.begin(115200);    
    /* Connects to AP */    
    WiFi.begin(SSID_NAME, SSID_PASS);    
         
    while (WiFi.status() != WL_CONNECTED) {    
      delay(500);    
      Serial.print(".");    
      }    
         
    WiFi.setAutoReconnect(true);    
    Serial.println(F("WiFi connected"));    
    Serial.println(F("IP address: "));    
    Serial.println(WiFi.localIP());    
         
    if (clientinfi.connect(HTTPSERVER, HTTPPORT)) {    
      Serial.println("connected to Ubidots cloud");    
    } else {    
      Serial.println("could not connect to Ubidots cloud");    
    }    
         
    }    
         
    void loop() {    
         
    // Space to store values to send    
    char payload[200];    
    char str_val_1[30];    
    char str_lat[30];    
    char str_lng[30];    
         
    /*---- Simulates the values of the sensors -----*/    
    float sensor_value_1 = random(0, 1000)*1.0;    
         
    /*---- Transforms the values of the sensors to char type -----*/    
         
    /* 4 is mininum width, 2 is precision; float value is copied onto str_val*/    
    dtostrf(sensor_value_1, 4, 2, str_val_1);    
         
    // Important: Avoid to send a very long char as it is very memory space costly, send small char arrays    
    sprintf(payload, "{\"");    
    sprintf(payload, "%s%s\":{\"value\":%s}", payload, VARIABLE_LABEL_1, str_val_1);    
    sprintf(payload, "%s}", payload);    
         
    /* Calls the Ubidots Function POST */    
    SendToInfiiot(payload);    
         
    delay(5000);    
    }

'''    
    This Example sends harcoded data to Ubidots using the request HTTP    
    library.    
         
    Please install the library using pip install requests    
         
    Made by Jose García @https://github.com/jotathebest/    
    '''    
         
    import requests    
    import random    
    import time    
         
    '''    
    global variables    
    '''    
         
    ENDPOINT = "infiiot.com"    
    DEVICE_LABEL = "weather-station"    
    VARIABLE_LABEL = "temperature"    
    TOKEN = ""    
    DELAY = 1  # Delay in seconds    
         
    def post_var(payload, url=ENDPOINT, device=DEVICE_LABEL, token=TOKEN):    
      try:    
          url = "http://{}/api/v1.0/update-device/{}".format(url, device)    
          headers = {"X-Auth-Token": token, "Content-Type": "application/json"}    
         
          attempts = 0    
          status_code = 400    
         
          while status_code >= 400 and attempts < 5:    
              print("[INFO] Sending data, attempt number: {}".format(attempts))    
              req = requests.post(url=url, headers=headers,    
                                  json=payload)    
              status_code = req.status_code    
              attempts += 1    
              time.sleep(1)    
         
          print("[INFO] Results:")    
          print(req.text)    
      except Exception as e:    
          print("[ERROR] Error posting, details: {}".format(e))    
         
    def main():    
      # Simulates sensor values    
        sensor_value = random.random() * 100    
         
      # Builds Payload and topíc    
        payload = {VARIABLE_LABEL: sensor_value}    
         
      # Sends data    
        post_var(payload)    
         
    if __name__ == "__main__":    
      while True:    
          main()    
          time.sleep(DELAY)

Send data-point with coordinates

curl -X POST -H "X-Auth-Token: BBFF-Rfcgaxns6HlVb155WA0RhSY85xNDmB" -H "Content-Type: application/json" -d '{"position": {"value" : 1, "timestamp": 1514808000000, "context":{"lat":-6.2, "lng":75.4}}}' https://infiiot.com/api/v1.0/devices/update-device

/*************************************************************************************************    
    * This Example sends harcoded data to Ubidots and serves as example for users that have devices    
    * based on ESP8266 chips    
    *    
    * This example is given AS IT IS without any warranty    
    *    
    * Made by Jose García., adapted from the original WiFiClient ESP8266 example    
    *************************************************************************************************/    
         
    /********************************    
    * Libraries included    
    *******************************/    
         
    #include <ESP8266WiFi.h>    
         
    /********************************    
    * Constants and objects    
    *******************************/    
    char const * SSID_NAME = "..."; // Put here your SSID name    
    char const * SSID_PASS = "...."; // Put here your password    
         
    char* TOKEN = ""; // Put here your TOKEN    
         
    char* DEVICE_LABEL = "truck"; // Your Device label    
         
    /* Put here your variable's labels*/    
    char const * VARIABLE_LABEL_1 = "position";    
         
    /* HTTP Settings */    
    char const * HTTPSERVER = "infiiot.com";    
    const int HTTPPORT = 80;    
    char const * USER_AGENT = "ESP8266";    
    char const * VERSION = "1.0";    
         
    WiFiClient clientinfi;    
         
    /********************************    
    * Auxiliar Functions    
    *******************************/    
         
    void SendToInfiiot(char* payload) {    
         
    int contentLength = strlen(payload);    
         
    /* Connecting the client */    
    clientinfi.connect(HTTPSERVER, HTTPPORT);    
         
    if (clientinfi.connected()) {    
      /* Builds the request POST - Please reference this link to know all the request's structures https://ubidots.com/docs/api/ */    
      clientinfi.print(F("POST /api/v1.0/update-device"));    
      clientinfi.print(DEVICE_LABEL);    
      clientinfi.print(F(" HTTP/1.1\r\n"));    
      clientinfi.print(F("Host: "));    
      clientinfi.print(HTTPSERVER);    
      clientinfi.print(F("\r\n"));    
      clientinfi.print(F("User-Agent: "));    
      clientinfi.print(USER_AGENT);    
      clientinfi.print(F("/"));    
      clientinfi.print(VERSION);    
      clientinfi.print(F("\r\n"));    
      clientinfi.print(F("X-Auth-Token: "));    
      clientinfi.print(TOKEN);    
      clientinfi.print(F("\r\n"));    
      clientinfi.print(F("Connection: close\r\n"));    
      clientinfi.print(F("Content-Type: application/json\r\n"));    
      clientinfi.print(F("Content-Length: "));    
      clientinfi.print(contentLength);    
      clientinfi.print(F("\r\n\r\n"));    
      clientinfi.print(payload);    
      clientinfi.print(F("\r\n"));    
         
      Serial.println(F("Making request to Ubidots:\n"));    
      Serial.print("POST /api/v1.0/update-device");    
      Serial.print(DEVICE_LABEL);    
      Serial.print(" HTTP/1.1\r\n");    
      Serial.print("Host: ");    
      Serial.print(HTTPSERVER);    
      Serial.print("\r\n");    
      Serial.print("User-Agent: ");    
      Serial.print(USER_AGENT);    
      Serial.print("/");    
      Serial.print(VERSION);    
      Serial.print("\r\n");    
      Serial.print("X-Auth-Token: ");    
      Serial.print(TOKEN);    
      Serial.print("\r\n");    
      Serial.print("Connection: close\r\n");    
      Serial.print("Content-Type: application/json\r\n");    
      Serial.print("Content-Length: ");    
      Serial.print(contentLength);    
      Serial.print("\r\n\r\n");    
      Serial.print(payload);    
      Serial.print("\r\n");    
    } else {    
      Serial.println("Connection Failed ubidots - Try Again");    
    }    
         
    /* Reach timeout when the server is unavailable */    
    int timeout = 0;    
    while (!clientinfi.available() && timeout < 5000) {    
      timeout++;    
      delay(1);    
      if (timeout >= 5000) {    
        Serial.println(F("Error, max timeout reached"));    
        break;    
      }    
    }    
         
    /* Reads the response from the server */    
    Serial.println(F("\nUbidots' Server response:\n"));    
    while (clientinfi.available()) {    
      char c = clientinfi.read();    
      Serial.print(c); // Uncomment this line to visualize the response on the Serial Monitor    
    }    
         
    /* Disconnecting the client */    
    clientinfi.stop();    
    }    
         
    /********************************    
    * Main Functions    
    *******************************/    
         
    void setup() {    
    Serial.begin(115200);    
    /* Connects to AP */    
    WiFi.begin(SSID_NAME, SSID_PASS);    
         
    while (WiFi.status() != WL_CONNECTED) {    
      delay(500);    
      Serial.print(".");    
      }    
         
    WiFi.setAutoReconnect(true);    
    Serial.println(F("WiFi connected"));    
    Serial.println(F("IP address: "));    
    Serial.println(WiFi.localIP());    
         
    if (clientinfi.connect(HTTPSERVER, HTTPPORT)) {    
      Serial.println("connected to Ubidots cloud");    
    } else {    
      Serial.println("could not connect to Ubidots cloud");    
    }    
         
    }    
         
    void loop() {    
         
    // Space to store values to send    
    char payload[200];    
    char str_val_1[30];    
    char str_lat[30];    
    char str_lng[30];    
         
    /*---- Simulates the values of the sensors -----*/    
    float sensor_value_1 = random(0, 1000)*1.0;    
    float lat = -6.256;    
    float lng = 75.236;    
         
    /*---- Transforms the values of the sensors to char type -----*/    
         
    /* 4 is mininum width, 2 is precision; float value is copied onto str_val*/    
    dtostrf(sensor_value_1, 4, 2, str_val_1);    
    dtostrf(lat, 4, 2, str_lat);    
    dtostrf(lng, 4, 2, str_lng);    
         
    // Important: Avoid to send a very long char as it is very memory space costly, send small char arrays    
    sprintf(payload, "{\"");    
    sprintf(payload, "%s%s\":{\"value\":%s", payload, VARIABLE_LABEL_1, str_val_1);    
    sprintf(payload, "%s,\"context\":{\"lat\":%s, \"lng\":%s}}", payload, str_lat, str_lng);    
    sprintf(payload, "%s}", payload);    
         
    /* Calls the Ubidots Function POST */    
    SendToInfiiot(payload);    
         
    delay(5000);    
         
    }

'''    
    This Example sends harcoded data to Ubidots using the request HTTP    
    library.    
         
    Please install the library using pip install requests    
         
    Made by Jose García @https://github.com/jotathebest/    
    '''    
         
    import requests    
    import random    
    import time    
         
    '''    
    global variables    
    '''    
         
    ENDPOINT = "infiiot.com"    
    DEVICE_LABEL = "truck"    
    VARIABLE_LABEL = "position"    
    TOKEN = "BBFF-YTP65d9ngV1useDm97rfZQ7j4QtijV"    
    DELAY = 1  # Delay in seconds    
         
    def post_var(payload, url=ENDPOINT, device=DEVICE_LABEL, token=TOKEN):    
      try:    
          url = "http://{}/api/v1.0/update-device/{}".format(url, device)    
          headers = {"X-Auth-Token": token, "Content-Type": "application/json"}    
         
          attempts = 0    
          status_code = 400    
         
          while status_code >= 400 and attempts < 5:    
              print("[INFO] Sending data, attempt number: {}".format(attempts))    
              req = requests.post(url=url, headers=headers,    
                                  json=payload)    
              status_code = req.status_code    
              attempts += 1    
              time.sleep(1)    
         
          print("[INFO] Results:")    
          print(req.text)    
      except Exception as e:    
          print("[ERROR] Error posting, details: {}".format(e))    
         
    def main():    
      # Simulates sensor values    
        sensor_value = random.random() * 100    
      lat = 6.101    
      lng = -71.293    
         
      # Builds Payload and topíc    
        payload = {VARIABLE_LABEL: {"value": sensor_value,    
                                  "context": {"lat": lat, "lng": lng}}    
                 }    
         
      # Sends data    
        post_var(payload)    
         
    if __name__ == "__main__":    
      while True:    
          main()    
          time.sleep(DELAY)

Send multiple data-points

curl -X POST -H "X-Auth-Token: BBFF-Rfcgaxns6HlVb155WA0RhSY85xNDmB" -H "Content-Type: application/json" -d '{"temperature": {"value": 27, "timestamp": 1514808000000}, "humidity": 55, "pressure": {"value": 78, "context":{name" : "John"}}}'    
    https://infiiot.com/api/v1.0/devices/update-device

/*************************************************************************************************    
    * This Example sends harcoded data to Ubidots and serves as example for users that have devices    
    * based on Arduino Ethernet shield    
    *    
    * This example is given AS IT IS without any warranty    
    *    
    * Made by Jose García., adapted from the original Arduino Ethernet example    
    *************************************************************************************************/    
         
    /********************************    
    * Libraries included    
    *******************************/    
         
    #include <ESP8266WiFi.h>    
         
    /********************************    
    * Constants and objects    
    *******************************/    
    char const * SSID_NAME = "..."; // Put here your SSID name    
    char const * SSID_PASS = "...."; // Put here your password    
         
    char* TOKEN = ""; // Put here your TOKEN    
         
    char* DEVICE_LABEL = "weather-station"; // Your Device label    
         
    /* Put here your variable's labels*/    
    char const * VARIABLE_LABEL_1 = "temperature";    
    char const * VARIABLE_LABEL_2 = "humidity";    
         
    /* HTTP Settings */    
    char const * HTTPSERVER = "infiiot.com";    
    const int HTTPPORT = 80;    
    char const * USER_AGENT = "ESP8266";    
    char const * VERSION = "1.0";    
         
    WiFiClient clientinfi;    
         
    /********************************    
    * Auxiliar Functions    
    *******************************/    
         
    void SendToInfiiot(char* payload) {    
         
    int contentLength = strlen(payload);    
         
    /* Connecting the client */    
    clientinfi.connect(HTTPSERVER, HTTPPORT);    
         
    if (clientinfi.connected()) {    
      /* Builds the request POST - Please reference this link to know all the request's structures https://ubidots.com/docs/api/ */    
      clientinfi.print(F("POST /api/v1.0/update-device"));    
      clientinfi.print(DEVICE_LABEL);    
      clientinfi.print(F(" HTTP/1.1\r\n"));    
      clientinfi.print(F("Host: "));    
      clientinfi.print(HTTPSERVER);    
      clientinfi.print(F("\r\n"));    
      clientinfi.print(F("User-Agent: "));    
      clientinfi.print(USER_AGENT);    
      clientinfi.print(F("/"));    
      clientinfi.print(VERSION);    
      clientinfi.print(F("\r\n"));    
      clientinfi.print(F("X-Auth-Token: "));    
      clientinfi.print(TOKEN);    
      clientinfi.print(F("\r\n"));    
      clientinfi.print(F("Connection: close\r\n"));    
      clientinfi.print(F("Content-Type: application/json\r\n"));    
      clientinfi.print(F("Content-Length: "));    
      clientinfi.print(contentLength);    
      clientinfi.print(F("\r\n\r\n"));    
      clientinfi.print(payload);    
      clientinfi.print(F("\r\n"));    
         
      Serial.println(F("Making request to Ubidots:\n"));    
      Serial.print("POST /api/v1.0/update-device");    
      Serial.print(DEVICE_LABEL);    
      Serial.print(" HTTP/1.1\r\n");    
      Serial.print("Host: ");    
      Serial.print(HTTPSERVER);    
      Serial.print("\r\n");    
      Serial.print("User-Agent: ");    
      Serial.print(USER_AGENT);    
      Serial.print("/");    
      Serial.print(VERSION);    
      Serial.print("\r\n");    
      Serial.print("X-Auth-Token: ");    
      Serial.print(TOKEN);    
      Serial.print("\r\n");    
      Serial.print("Connection: close\r\n");    
      Serial.print("Content-Type: application/json\r\n");    
      Serial.print("Content-Length: ");    
      Serial.print(contentLength);    
      Serial.print("\r\n\r\n");    
      Serial.print(payload);    
      Serial.print("\r\n");    
    } else {    
      Serial.println("Connection Failed ubidots - Try Again");    
    }    
         
    /* Reach timeout when the server is unavailable */    
    int timeout = 0;    
    while (!clientinfi.available() && timeout < 5000) {    
      timeout++;    
      delay(1);    
      if (timeout >= 5000) {    
        Serial.println(F("Error, max timeout reached"));    
        break;    
      }    
    }    
         
    /* Reads the response from the server */    
    Serial.println(F("\nUbidots' Server response:\n"));    
    while (clientinfi.available()) {    
      char c = clientinfi.read();    
      Serial.print(c); // Uncomment this line to visualize the response on the Serial Monitor    
    }    
         
    /* Disconnecting the client */    
    clientinfi.stop();    
    }    
         
    /********************************    
    * Main Functions    
    *******************************/    
         
    void setup() {    
    Serial.begin(115200);    
    /* Connects to AP */    
    WiFi.begin(SSID_NAME, SSID_PASS);    
         
    while (WiFi.status() != WL_CONNECTED) {    
      delay(500);    
      Serial.print(".");    
      }    
         
    WiFi.setAutoReconnect(true);    
    Serial.println(F("WiFi connected"));    
    Serial.println(F("IP address: "));    
    Serial.println(WiFi.localIP());    
         
    if (clientinfi.connect(HTTPSERVER, HTTPPORT)) {    
      Serial.println("connected to Ubidots cloud");    
    } else {    
      Serial.println("could not connect to Ubidots cloud");    
    }    
         
    }    
         
    void loop() {    
         
    // Space to store values to send    
    char payload[200];    
    char str_val_1[30];    
    char str_val_2[30];    
         
    /*---- Simulates the values of the sensors -----*/    
    float sensor_value_1 = random(0, 1000)*1.0;    
    float sensor_value_2 = random(0, 1000)*1.0;    
         
    /*---- Transforms the values of the sensors to char type -----*/    
         
    /* 4 is mininum width, 2 is precision; float value is copied onto str_val*/    
    dtostrf(sensor_value_1, 4, 2, str_val_1);    
    dtostrf(sensor_value_2, 4, 2, str_val_2);    
         
    // Important: Avoid to send a very long char as it is very memory space costly, send small char arrays    
    sprintf(payload, "{\"");    
    sprintf(payload, "%s%s\":%s", payload, VARIABLE_LABEL_1, str_val_1);    
    sprintf(payload, "%s,\"%s\":%s", payload, VARIABLE_LABEL_2, str_val_2);    
    sprintf(payload, "%s}", payload);    
         
    /* Calls the Ubidots Function POST */    
    SendToInfiiot(payload);    
         
    delay(5000);    
         
    }

'''    
    This Example sends harcoded data to Ubidots using the request HTTP    
    library.    
         
    Please install the library using pip install requests    
         
    Made by Jose García @https://github.com/jotathebest/    
    '''    
         
    import requests    
    import random    
    import time    
         
    '''    
    global variables    
    '''    
         
    ENDPOINT = "infiiot.com"    
    DEVICE_LABEL = "weather-station"    
    VARIABLE_LABEL_1 = "temperature"    
    VARIABLE_LABEL_2 = "humidity"    
    TOKEN = "..."    
    DELAY = 1  # Delay in seconds    
         
    def post_var(payload, url=ENDPOINT, device=DEVICE_LABEL, token=TOKEN):    
      try:    
          url = "http://{}/api/v1.0/update-device/{}".format(url, device)    
          headers = {"X-Auth-Token": token, "Content-Type": "application/json"}    
         
          attempts = 0    
          status_code = 400    
         
          while status_code >= 400 and attempts < 5:    
              print("[INFO] Sending data, attempt number: {}".format(attempts))    
              req = requests.post(url=url, headers=headers,    
                                  json=payload)    
              status_code = req.status_code    
              attempts += 1    
              time.sleep(1)    
         
          print("[INFO] Results:")    
          print(req.text)    
      except Exception as e:    
          print("[ERROR] Error posting, details: {}".format(e))    
         
    def main():    
      # Simulates sensor values    
        sensor_value_1 = random.random() * 100    
      sensor_value_2 = random.random() * 100    
         
      # Builds Payload and topíc    
        payload = {VARIABLE_LABEL_1: sensor_value_1,    
                 VARIABLE_LABEL_2: sensor_value_2    
                }    
         
      # Sends data    
        post_var(payload)    
         
    if __name__ == "__main__":    
      while True:    
          main()    
          time.sleep(DELAY)

## Send data to a variable

POST https://infiiot.com/api/v1.0/update-variable

Send a Single data-point

curl -X POST -H "X-Auth-Token: BBFF-Rfcgaxns6HlVb155WA0RhSY85xNDmB" -H "Content-Type: application/json" -d '{"value": 27}' curl -X POST -H "X-Auth-Token: BBFF-Rfcgaxns6HlVb155WA0RhSY85xNDmB" -H "Content-Type: application/json" -d '[{"value": 27, "timestamp":1514808000000}, {"value": 12, "timestamp":1514808900000}]' https://infiiot.com/api/v1.0/update-variable

/*************************************************************************************************    
    * This Example sends harcoded data to Ubidots and serves as example for users that have devices    
    * based on ESP8266 chips    
    *    
    * This example is given AS IT IS without any warranty    
    *    
    * Made by Jose García @https://github.com/jotathebest/ ,    
    * adapted from the original WiFiClient ESP8266 example    
    *************************************************************************************************/    
         
    /********************************    
    * Libraries included    
    *******************************/    
         
    #include <ESP8266WiFi.h>    
         
    /********************************    
    * Constants and objects    
    *******************************/    
    char const * SSID_NAME = "...."; // Put here your SSID name    
    char const * SSID_PASS = "...."; // Put here your password    
         
    char* TOKEN = "...."; // Put here your TOKEN    
         
    char* DEVICE_LABEL = "weather-station"; // Your Device label    
         
    /* Put here your variable's labels*/    
    char const * VARIABLE_LABEL_1 = "temperature";    
         
    /* HTTP Settings */    
    char const * HTTPSERVER = "infiiot.com";    
    const int HTTPPORT = 80;    
    char const * USER_AGENT = "ESP8266";    
    char const * VERSION = "1.0";    
         
    WiFiClient clientUbi;    
         
    /********************************    
    * Auxiliar Functions    
    *******************************/    
         
    void SendToUbidots(char* payload) {    
         
    int contentLength = strlen(payload);    
         
    /* Connecting the client */    
    clientUbi.connect(HTTPSERVER, HTTPPORT);    
         
    if (clientUbi.connected()) {    
      /* Builds the request POST - Please reference this link to know all the request's structures https://ubidots.com/docs/api/ */    
      clientUbi.print(F("POST /api/v1.6/devices/"));    
      clientUbi.print(DEVICE_LABEL);    
      clientUbi.print(F(" HTTP/1.1\r\n"));    
      clientUbi.print(F("Host: "));    
      clientUbi.print(HTTPSERVER);    
      clientUbi.print(F("\r\n"));    
      clientUbi.print(F("User-Agent: "));    
      clientUbi.print(USER_AGENT);    
      clientUbi.print(F("/"));    
      clientUbi.print(VERSION);    
      clientUbi.print(F("\r\n"));    
      clientUbi.print(F("X-Auth-Token: "));    
      clientUbi.print(TOKEN);    
      clientUbi.print(F("\r\n"));    
      clientUbi.print(F("Connection: close\r\n"));    
      clientUbi.print(F("Content-Type: application/json\r\n"));    
      clientUbi.print(F("Content-Length: "));    
      clientUbi.print(contentLength);    
      clientUbi.print(F("\r\n\r\n"));    
      clientUbi.print(payload);    
      clientUbi.print(F("\r\n"));    
         
      Serial.println(F("Making request to Ubidots:\n"));    
      Serial.print("POST /api/v1.6/devices/");    
      Serial.print(DEVICE_LABEL);    
      Serial.print(" HTTP/1.1\r\n");    
      Serial.print("Host: ");    
      Serial.print(HTTPSERVER);    
      Serial.print("\r\n");    
      Serial.print("User-Agent: ");    
      Serial.print(USER_AGENT);    
      Serial.print("/");    
      Serial.print(VERSION);    
      Serial.print("\r\n");    
      Serial.print("X-Auth-Token: ");    
      Serial.print(TOKEN);    
      Serial.print("\r\n");    
      Serial.print("Connection: close\r\n");    
      Serial.print("Content-Type: application/json\r\n");    
      Serial.print("Content-Length: ");    
      Serial.print(contentLength);    
      Serial.print("\r\n\r\n");    
      Serial.print(payload);    
      Serial.print("\r\n");    
    } else {    
      Serial.println("Connection Failed ubidots - Try Again");    
    }    
         
    /* Reach timeout when the server is unavailable */    
    int timeout = 0;    
    while (!clientUbi.available() && timeout < 5000) {    
      timeout++;    
      delay(1);    
      if (timeout >= 5000) {    
        Serial.println(F("Error, max timeout reached"));    
        break;    
      }    
    }    
         
    /* Reads the response from the server */    
    Serial.println(F("\nUbidots' Server response:\n"));    
    while (clientUbi.available()) {    
      char c = clientUbi.read();    
      Serial.print(c); // Uncomment this line to visualize the response on the Serial Monitor    
    }    
         
    /* Disconnecting the client */    
    clientUbi.stop();    
    }    
         
    /********************************    
    * Main Functions    
    *******************************/    
         
    void setup() {    
    Serial.begin(115200);    
    /* Connects to AP */    
    WiFi.begin(SSID_NAME, SSID_PASS);    
         
    while (WiFi.status() != WL_CONNECTED) {    
      delay(500);    
      Serial.print(".");    
      }    
         
    WiFi.setAutoReconnect(true);    
    Serial.println(F("WiFi connected"));    
    Serial.println(F("IP address: "));    
    Serial.println(WiFi.localIP());    
         
    if (clientUbi.connect(HTTPSERVER, HTTPPORT)) {    
      Serial.println("connected to Ubidots cloud");    
    } else {    
      Serial.println("could not connect to Ubidots cloud");    
    }    
         
    }    
         
    void loop() {    
         
    // Space to store values to send    
    char payload[200];    
    char str_val_1[30];    
         
    /*---- Simulates the values of the sensors -----*/    
    float sensor_value_1 = random(0, 1000)*1.0;    
         
    /*---- Transforms the values of the sensors to char type -----*/    
         
    /* 4 is mininum width, 2 is precision; float value is copied onto str_val*/    
    dtostrf(sensor_value_1, 4, 2, str_val_1);    
         
    // Important: Avoid to send a very long char as it is very memory space costly, send small char arrays    
         
    sprintf(payload, "{\"");    
    sprintf(payload, "%s%s\":{\"value\":%s}", payload, VARIABLE_LABEL_1, str_val_1);    
    sprintf(payload, "%s}", payload);    
         
    /* Calls the Ubidots Function POST */    
    SendToUbidots(payload);    
         
    delay(5000);    
         
    }

'''    
    This Example sends harcoded data to Ubidots using the requests HTTP    
    library.    
         
    Please install the library using pip install requests    
         
    Made by Jose García @https://github.com/jotathebest/    
    '''    
         
    import requests    
    import random    
    import time    
         
    '''    
    global variables    
    '''    
         
    ENDPOINT = "things.ubidots.com"    
    DEVICE_LABEL = "weather-station"    
    VARIABLE_LABEL = "temperature"    
    TOKEN = ""    
    DELAY = 1  # Delay in seconds    
         
    def post_var(payload, url=ENDPOINT, device=DEVICE_LABEL, token=TOKEN):    
      try:    
          url = "http://{}/api/v1.6/devices/{}".format(url, device)    
          headers = {"X-Auth-Token": token, "Content-Type": "application/json"}    
         
          attempts = 0    
          status_code = 400    
         
          while status_code >= 400 and attempts < 5:    
              print("[INFO] Sending data, attempt number: {}".format(attempts))    
              req = requests.post(url=url, headers=headers,    
                                  json=payload)    
              status_code = req.status_code    
              attempts += 1    
              time.sleep(1)    
         
          print("[INFO] Results:")    
          print(req.text)    
      except Exception as e:    
          print("[ERROR] Error posting, details: {}".format(e))    
         
    def main():    
      # Simulates sensor values    
        sensor_value = random.random() * 100    
         
      # Builds Payload and topíc    
        payload = {VARIABLE_LABEL: sensor_value}    
         
      # Sends data    
        post_var(payload)    
         
    if __name__ == "__main__":    
      while True:    
          main()    
          time.sleep(DELAY)

Send multiple data-points

curl -X POST -H "X-Auth-Token: BBFF-Rfcgaxns6HlVb155WA0RhSY85xNDmB" -H "Content-Type: application/json" -d '[{"value": 27, "timestamp":1514808000000}, {"value": 12, "timestamp":1514808900000}]' https://infiiot.com/api/v1.0/update-variable

/*************************************************************************************************    
    * This Example sends harcoded data to Ubidots and serves as example for users that have devices    
    * based on ESP8266 chips    
    *    
    * This example is given AS IT IS without any warranty    
    *    
    * Made by Jose García @https://github.com/jotathebest/ ,    
    * adapted from the original WiFiClient ESP8266 example    
    *************************************************************************************************/    
         
    /********************************    
    * Libraries included    
    *******************************/    
         
    #include <ESP8266WiFi.h>    
         
    /********************************    
    * Constants and objects    
    *******************************/    
    char const * SSID_NAME = "...."; // Put here your SSID name    
    char const * SSID_PASS = "...."; // Put here your password    
         
    char* TOKEN = "...."; // Put here your TOKEN    
         
    char* DEVICE_LABEL = "truck"; // Your Device label    
         
    /* Put here your variable's labels*/    
    char const * VARIABLE_LABEL_1 = "speed";    
         
    /* HTTP Settings */    
    char const * HTTPSERVER = "infiiot.com";    
    const int HTTPPORT = 80;    
    char const * USER_AGENT = "ESP8266";    
    char const * VERSION = "1.0";    
         
    WiFiClient clientUbi;    
         
    /********************************    
    * Auxiliar Functions    
    *******************************/    
         
    void SendToUbidots(char* payload) {    
         
    int contentLength = strlen(payload);    
         
    /* Connecting the client */    
    clientUbi.connect(HTTPSERVER, HTTPPORT);    
         
    if (clientUbi.connected()) {    
      /* Builds the request POST - Please reference this link to know all the request's structures https://ubidots.com/docs/api/ */    
      clientUbi.print(F("POST /api/v1.6/devices/"));    
      clientUbi.print(DEVICE_LABEL);    
      clientUbi.print(F("/"));    
      clientUbi.print(VARIABLE_LABEL_1);    
      clientUbi.print(F("/values HTTP/1.1\r\n"));    
      clientUbi.print(F("Host: "));    
      clientUbi.print(HTTPSERVER);    
      clientUbi.print(F("\r\n"));    
      clientUbi.print(F("User-Agent: "));    
      clientUbi.print(USER_AGENT);    
      clientUbi.print(F("/"));    
      clientUbi.print(VERSION);    
      clientUbi.print(F("\r\n"));    
      clientUbi.print(F("X-Auth-Token: "));    
      clientUbi.print(TOKEN);    
      clientUbi.print(F("\r\n"));    
      clientUbi.print(F("Connection: close\r\n"));    
      clientUbi.print(F("Content-Type: application/json\r\n"));    
      clientUbi.print(F("Content-Length: "));    
      clientUbi.print(contentLength);    
      clientUbi.print(F("\r\n\r\n"));    
      clientUbi.print(payload);    
      clientUbi.print(F("\r\n"));    
         
      Serial.println(F("Making request to Ubidots:\n"));    
      Serial.print(F("POST /api/v1.6/devices/"));    
      Serial.print(DEVICE_LABEL);    
      Serial.print(F("/"));    
      Serial.print(VARIABLE_LABEL_1);    
      Serial.print(F("/values HTTP/1.1\r\n"));    
      Serial.print("Host: ");    
      Serial.print(HTTPSERVER);    
      Serial.print("\r\n");    
      Serial.print("User-Agent: ");    
      Serial.print(USER_AGENT);    
      Serial.print("/");    
      Serial.print(VERSION);    
      Serial.print("\r\n");    
      Serial.print("X-Auth-Token: ");    
      Serial.print(TOKEN);    
      Serial.print("\r\n");    
      Serial.print("Connection: close\r\n");    
      Serial.print("Content-Type: application/json\r\n");    
      Serial.print("Content-Length: ");    
      Serial.print(contentLength);    
      Serial.print("\r\n\r\n");    
      Serial.print(payload);    
      Serial.print("\r\n");    
      /* Reach timeout when the server is unavailable */    
      int timeout = 0;    
      while (!clientUbi.available() && timeout < 5000) {    
        timeout++;    
        delay(1);    
        if (timeout >= 5000) {    
          Serial.println(F("Error, max timeout reached"));    
          break;    
        }    
      }    
         
      /* Reads the response from the server */    
      Serial.println(F("\nUbidots' Server response:\n"));    
      while (clientUbi.available()) {    
        char c = clientUbi.read();    
        Serial.print(c); // Uncomment this line to visualize the response on the Serial Monitor    
      }    
         
    } else {    
      Serial.println("Connection Failed ubidots - Try Again");    
    }    
         
    /* Disconnecting the client */    
    clientUbi.stop();    
    }    
         
    /********************************    
    * Main Functions    
    *******************************/    
         
    void setup() {    
    Serial.begin(115200);    
    /* Connects to AP */    
    WiFi.begin(SSID_NAME, SSID_PASS);    
         
    while (WiFi.status() != WL_CONNECTED) {    
      delay(500);    
      Serial.print(".");    
      }    
         
    WiFi.setAutoReconnect(true);    
    Serial.println(F("WiFi connected"));    
    Serial.println(F("IP address: "));    
    Serial.println(WiFi.localIP());    
         
    if (clientUbi.connect(HTTPSERVER, HTTPPORT)) {    
      Serial.println("connected to Ubidots cloud");    
    } else {    
      Serial.println("could not connect to Ubidots cloud");    
    }    
         
    }    
         
    void loop() {    
         
    // Space to store values to send    
    char payload[200];    
    char str_val_1[30];    
    char str_val_2[30];    
         
    /*---- Simulates the values of the sensors -----*/    
    float sensor_value_1 = random(0, 1000)*1.0;    
    float sensor_value_2 = random(0, 1000)*1.0;    
         
    /*---- Timestamps in seconds of the dots -----*/    
    long timestamp_1 = 1535555740L - (60 * 5);  // Dot with 5 minutes in the past    
    long timestamp_2 = 1535555740L;    
         
    /*---- Transforms the values of the sensors to char type -----*/    
    /* 4 is mininum width, 2 is precision; float value is copied onto str_val*/    
    dtostrf(sensor_value_1, 4, 2, str_val_1);    
    dtostrf(sensor_value_2, 4, 2, str_val_2);    
         
    // Important: Avoid to send a very long char as it is very memory space costly, send small char arrays    
    sprintf(payload, "[{\"value\":%s, \"timestamp\": %ld000},", str_val_1, timestamp_1);\    
    sprintf(payload, "%s{\"value\":%s, \"timestamp\": %ld000}", payload, str_val_2, timestamp_2);\    
    sprintf(payload, "%s]", payload);    
         
    /* Calls the Ubidots Function POST */    
    SendToUbidots(payload);    
         
    delay(5000);    
         
    }

'''    
    This Example sends harcoded data to Ubidots using the requests    
    library.    
         
    Please install the library using pip install requests    
         
    Made by Jose García @https://github.com/jotathebest/    
    '''    
         
    import requests    
    import random    
    import time    
         
    '''    
    global variables    
    '''    
         
    ENDPOINT = "industrial.api.ubidots.com"    
    DEVICE_LABEL = "weather-station"    
    VARIABLE_LABEL = "temperature"    
    TOKEN = ""    
    DELAY = 1  # Delay in seconds    
         
    def post_var(payload, url=ENDPOINT, device=DEVICE_LABEL,    
               variable=VARIABLE_LABEL, token=TOKEN):    
      try:    
          url = "http://{}/api/v1.6/devices/{}/{}/values".format(url,    
                                                                 device,    
                                                                 variable)    
          headers = {"X-Auth-Token": token, "Content-Type": "application/json"}    
         
          attempts = 0    
          status_code = 400    
         
          while status_code >= 400 and attempts < 5:    
              print("[INFO] Sending data, attempt number: {}".format(attempts))    
              req = requests.post(url=url, headers=headers,    
                                  json=payload)    
              status_code = req.status_code    
              attempts += 1    
              time.sleep(1)    
         
          print("[INFO] Results:")    
          print(req.text)    
      except Exception as e:    
          print("[ERROR] Error posting, details: {}".format(e))    
         
    def main():    
      # Simulates sensor values    
        sensor_value_1 = random.random() * 100    
      sensor_value_2 = random.random() * 100    
         
      # Creates timestamps with 5 and 10 min late    
        t_late_5min = int(time.time() * 1000) - 60 * 5    
      t_late_10min = int(time.time() * 1000) - 60 * 10    
         
      # Builds Payload and topíc    
        payload = [{"value": sensor_value_1, "timestamp": t_late_5min},\    
                 {"value": sensor_value_2, "timestamp": t_late_10min}\    
              ]    
         
      # Sends data    
        post_var(payload)    
         
    if __name__ == "__main__":    
      while True:    
          main()    
          time.sleep(DELAY)

# Retrieve data

Using our APIs you can retrive either multiple values or single data-point values.

Typical example of retrieving data is when you need to control fan, pump or your garage door from
Platform

If you are looking for more advanced API capabilities, such as retrieving a specific number of values
from device, retrieving data with exponential smoothing or data with pagination and, many more, please
refer to Infi-IoT Documentation-hardware.

### Request structure:

GET /api/v1.6/devices/{DEVICE_LABEL}/{VARIABLE_LABEL}/values HTTP/1.1

Host: {Host}

User-Agent: {USER_AGENT}

X-Auth-Token: {TOKEN}

Content-Type: application/json

### Expected Response:

HTTP/1.1 200 OK

Server: nginx

Date: Tue, 04 Sep 2018 22:35:06 GMT

Content-Type: application/json

Transfer-Encoding: chunked

Vary: Cookie

Allow: GET, POST, HEAD, OPTIONS

{PAYLOAD_LENGTH_IN_HEXADECIMAL}

{"count": true, "previous":
null, "results":
[{PAYLOAD_WITH_VALUES}], "next":
{URL_WITH_ADDITIONAL_VALUES}}

0

In the path specified above, replace the {DEVICELABEL} and {VARIABLE_LABEL} keys with the unique labels
of the Ubidots device and variable you desired to retrieve data from. This _path returns by default the
last 50 values stored of your
variable. You can manage the number of dots to retrieve using the page_size parameter in your path:

/api/v1.6/devices/{DEVICE_LABEL}/{VARIABLE_LABEL}/values/?page_size=2 The path above retrieves the last 2
dots of your variable.

Ubidots API also allows you to filter the values to be retrieved by date range, number of dots, and more.
If you desire to go deep into this kind of requests, please check the Ubidots Software REST API
Reference.

# Response Codes

Response codes help users know the status and errors for the requests made to the server. Infi-IoT
REST-API uses the following request status code when you make an HTTP request:

| Code | Detail |
| --- | --- |
| 200 | Ok -- Successful request. |
| 201 | Created -- Successful request + an item (device or variable) was created. |
| 202 | Accepted -- The request has been accepted for processing, but the processing has not been<br> completed. |
| 204 | One of the fields is incorrect and the request is not saved -- Please verify it's a valid<br> JSON string and that the fields are the ones expected by the endpoint (string, object, or<br> float). |
| 400 | Bad Request -- Error due to an invalid body in your request. Please verify it's a valid JSON<br> string and that the fields are the ones expected by the endpoint (string, object, or float). |
| 401 | Invalid API key -- Please verify your API Key. |
| 402 | Payment required -- Please verify your balance. |
| 403 | Forbidden -- This token is not valid. Please verify your token. |
| 404 | Not Found -- We couldn’t find the URL you're trying to access. This might be due to a wrong<br> device label or ID, a wrong variable label or ID, or simply a typo in the URL of the request. |
| 405 | Method Not Allowed -- This API endpoint does not accept the method used. Check our API docs to<br> see the allowed methods. |
| 415 | Unsupported media type -- The payload is in a format not supported by this method on the target<br> resource. |
| 420 | You have exceeded your API limits -- To upgrade your API limits contacts the Ubidots team |
| 423 | Device does not receive data because it is disabled |
| 429 | Too many requests -- Many requests sent in a given amount of time ("rate limiting") |
| 50 | Internal Error -- We're having issues with our servers. |

# Limits

Configuration updates are limited to 1 update per second, per device. However, for best results, device
configuration should be updated much less often — at most, once every 5 seconds.

The update rate is calculated as the time between the most recent server acknowledgment and the next
update request.

# MQTT

MQTT (Message Queuing Telemetry Transport) is lightweight messaging protocol that was developed by IBM
and first released in 1999. It uses the publish/subscribe messaging scheme and translates messages
between devices, servers, and applications.

Main features of MQTT are:

- Small code footprint (to make it easy to implement in small devices)

- Low power consumption

- Low bandwidth consumption

- Low latency

- Use of a publish/subscribe (“pub/sub”) pattern

## MQTT Architecture

The connected devices within the MQTT protocol are referred to as “clients,” which communicate with a
server refered as the “broker.” The broker handles the task of message transmission between clients.

Whenever a client (known as “publisher”) wants to distribute information, it'll publish to a specific
topic, the broker then sends this information to any clients that have subscribed to that topic (known
as “subscribers”).

The publisher don't need any data on the number or the locations of subscribers. In turn, subscribers
don't need any data about the publisher. Any client may be a publisher, subscriber, or both. The
clients are typically not conscious
of one another , only of the broker that perform as the intermediary. This setup is popularly known as
the “pub/sub model.”

## MQTT messages

When a client wants to send data to the broker, this is often referred to as a “publish.” When a client
wants to receive data from the broker, it'll “subscribe” to a topic or topics. When a client
subscribes to a particular topic,
it'll receive all messages published thereon topic going forward.

## Quality of Service(QOS)

Along with the message, the publisher also sends a QoS (Quality of Service) level which defines the
guarantee of delivery for the message. These QoS levels are as follows:

- QoS 0, delivered at most once When the message is published, the broker will only
receive the message “at most once.” This level should not be used for mission-critical information
since it carries the risk that the
subscribers will not receive the message.

- QoS 1, delivered at least once: The publisher continues to resend the message until
it receives an acknowledgment from the broker regarding the particular message. In other words, it’s
more important that the message
is received than it is to ensure it is only received once. This is probably the most commonly used
QoS level.

- QoS 2, delivered exactly once The publisher and broker work together to ensure the
broker will receive and act on a message exactly once. This requires some additional overhead in the
form of a four-part handshake.
Although this is the safest QoS level, it is also the slowest and therefore only used when
necessary.

Infi-IoT does not support QoS 2. Publishing QoS 2 messages closes the connection. Subscribing to a
predefined topic with QoS 2 downgrades the QoS level to QoS 1.

QoS 0 and 1 function as follows in Infi-IoT Platform:

- A PUBLISH message with QoS 1 will be acknowledged by the PUBACK message after it has been
successfully sent to Infi-IoT Platform Pub/Sub.

- PUBLISH messages with QoS 0 do not require PUBACK responses, and may be dropped if there is any
jitter along the message delivery path (for example, if Infi-IoT Platform Pub/Sub is temporarily
unavailable).

- The Infi-IoT Platform MQTT bridge maintains a small buffer of undelivered messages in order to
retry them. If the buffer becomes full, the message with QoS 1 may be dropped and a PUBACK message
will not be sent to the client. The client
is expected to resend the message.

MQTT

cURL ESP Python

![infiiot technologies logo along with infiiot iot platform logo](https://infiiot.com/static/images/logo-alt.png)

###### Services & Products

- Products & Design

- Industry Consultancy

- Infi-IOT Platform

- Web Development Services

- View Products!

###### Resources

- Getting Started

- Documentation- Hardware

- Documentation- Software

- Community Forum

- Blog

- Help Centre

- FAQs

###### Company

- About

- Contact Us

- Features

- Pricing

- Privacy Policy

- Terms of Service

Copyright © 2020
infiiot.com